Knight Campus BioFoundry Facility

User Handbook

Facility Guidelines, Laboratory Safety Plan (LSP), Training Guide and Lone Worker Policy

Revision: Rev. 4 – 04-22-26

Knight Campus – University of Oregon

Emergency Procedures and Incident Reporting

Report all accidents, emergencies, spills, exposures, injuries, and equipment failures immediately

Know locations of exits, eye wash, showers, fire extinguishers, spill kits

SDS/MSDS sheets near lab entry

Biological Spills: secure area, follow SOP, notify staff/EHS

Exposure Incidents: wash area, seek medical attention, complete report

Emergency Contacts:

911 (Emergencies)

UO Police: 541-346-2919

UO Facilities Dispatch: 541-346-2319

EHS duty phone number (for biological spills):  541-954-3605

BioFoundry Engineer: Raquel Simão Gurge – raquelsg@uoregon.edu – 541-346-7176

Building Facilities Director: Greg Normandin – gregn@uoregon.edu – 541-346-7052

Core Facilities Director: Kurt Langworthy – klangwor@uoregon.edu – 541-346-3660

1. Purpose and Scope	2

2. Area Configuration	3

Live Cell Incubation Area (Room 117A)	3

Live Cell Manipulation Area (Rooms 117 & 119)	3

Bio Characterization Area (Rooms 115, 115B, 115C, 115D)	4

Shared Support Area (Room 121)	4

3. Training, Access and Authorization	4

Required Training to Use FacilityTrainer	4

Authorization	5

4. Personal Protective Equipment (PPE)	5

Minimum PPE	6

Task‑Dependent PPE	6

5. Facility Entry and Exit Procedures	6

6. Biosafety Practices	7

Cell Culture Best Practices:	7

Materials Returning to Incubators and Fridges:	7

Incubator Guidelines:	8

Consumables Tracking:	8

Shared Responsibility:	8

Mandatory Core Equipment Training:	8

Proper Material Labeling:	8

7. Biological Materials Handling	8

8. Chemical Handling	9

9. Tool and Equipment Use	9

10. Waste Management and Decontamination	10

11. Housekeeping and Laboratory Etiquette	10

12. After‑Hours Use and Lone Worker Policy	11

Permitted Activities (with PI Approval)	11

Activities We Strongly Discourage:	11

Buddy System	11

13. Training, Records, and Documentation	11

14. Enforcement	11

15. Violations and Enforcement	12

# 1. Purpose and Scope

This document outlines access requirements, operational expectations, biosafety practices, and emergency procedures for the BSL‑2 BioFoundry facility. The facility supports biological sample characterization, analytical assays, and associated instrumentation under Biosafety Level 2 containment. These guidelines apply to faculty, students, staff, visiting scientists, and industry partners. All work must comply with University Environmental Health & Safety (EHS), Institutional Biosafety Committee (IBC), CDC/NIH BMBL guidance, and applicable federal, state, and local regulations.

# 2. Area Configuration

The BioFoundry is organized into functional areas designed to maintain biosafety, optimize workflow, and protect shared equipment and biological materials.

## Live Cell Incubation Area (Room 117A)

- Maintains and incubates viable cell cultures under controlled conditions.

- Supports CO2 incubators and culture storage systems.

- Permitted activities:

  - Incubation and storage of actively growing mammalian or low-contamination-risk cell cultures

  - Temporary placement of culture plates, flasks, or vessels in incubators

  - Routine maintenance (e.g., medium exchange)

  - Visual monitoring using an inverted microscope

  - Short-term handling to maintain cell viability

Restrictions: Only low-contamination-risk mammalian cells are allowed; bacteria, yeast, or other microorganisms are not permitted in this area. Please ask the BioFoundry Engineer for details incubating bacteria (and other cell types) BEFORE bringing cells to the facility.

## Live Cell Manipulation Area (Rooms 117 & 119)

- Dedicated to sterile handling and experimental manipulation of viable cells.

- Permitted activities:

  - Routine cell culture procedures (passaging, plating, expansion)

  - Genetic manipulation (transfection, viral vector work, if approved)

  - Sterile preparation of cultures

  - Cell sorting and preparation for cytometry workflows

  - Live-cell imaging and assays requiring viable cells

  - Experimental manipulation with biomaterials or devices

## Bio Characterization Area (Rooms 115, 115B, 115C, 115D)

- Dedicated to analyzing and characterizing biological samples and biomaterials.

- Permitted activities:

  - Microscopy and imaging of biological samples

  - Biochemical and molecular assays

  - Biomaterial and engineered construct characterization

  - Mechanobiology and mechanical testing experiments

  - Analysis of fixed/inactivated bacterial and yeast samples

- Important Note: Only fixed or inactivated microorganisms, including bacteria and yeast, are allowed in this area. Live microbial cultures are strictly prohibited unless approved

## Shared Support Area (Room 121)

- Provides general laboratory resources and infrastructure.

- Permitted activities:

  - Storage of reagents in shared refrigerators/freezers

  - Organization of glassware and laboratory supplies

  - Access to shared equipment (autoclaves, etc.)

  - Preparation of general laboratory materials

Note: This area is not for active experimental manipulation of biological materials.

Restrictions: Bacteria, yeast, or other microorganisms are not permitted unless they are genetically inactivated or fully inactivated by approved SOPs.

# 3. Training, Access and Authorization

## Required Training to Use Facility				Trainer

- Lab Safety and Hazardous Waste Management (Annually)	UO MyTrack/EHS

- Bloodborne Pathogens Online Training  (Annually)		UO MyTrack

- BSL‑2 Biosafety Training (when applicable)			UO MyTrack/EHS

- BioFoundry Facility Orientation 				BioFoundry Staff

- Instrument‑specific training for each instrument used 	BioFoundry Staff

*Register for trainings here: https://uomytrack.pageuppeople.com/

NEMO Account

Nemo account is required for scheduling training and equipment, logging usage, checking out consumables, and tracking service activities.

NEMO-KC Cores (https://nemo.uoregon.edu)

NEMO - User Account Setup (form link available here: https://knightcampus.uoregon.edu/facilities/core-facilities/user)

- Using equipment without recording the session is prohibited.

- All users must scan into the Nemo Kisok upon entering the lab.

All users must complete required institutional training prior to access.

## Authorization

- Users must be approved by BioFoundry Staff or the Core Facilities Director to gain access to the BioFoundry.

  - This authorization is required for all individuals entering the BioFoundry, including visitors or personnel who are only accompanying lab members and not performing experiments.

Visitor Policy:

- Pre-approved by staff and escorted at all times.

- Visitors may not use equipment or handle chemicals.

Building Policy:

- All users must follow Knight Campus building policies

# 4. Personal Protective Equipment (PPE)

## Minimum PPE

- Fully closed-toe shoes

- Long pants or equivalent coverage

- Laboratory coat (fluid-resistant required)

  - Bio Characterization – White coat

  - Cell incubation and manipulation – Blue coat

- Safety glasses or goggles

## Task‑Dependent PPE

- Face shields or splash goggles

- Double-gloving

- Disposable sleeve covers

- Respiratory protection (fit-tested if approved)

Remove PPE before leaving the lab; follow facility disposal/storage procedures.

- Any sign of contamination on lab coats (e.g., splashes, contact with contaminated areas, or getting wet) requires immediate replacement.

- Place the contaminated lab coat in the designated container for laundering.

- If a visitor borrowed a lab coat, ensure it is placed in the laundry bin after use.

- Change lab coats according to the specific area requirements to maintain biosafety.

Moving Between Laboratory Areas

- To prevent cross-contamination between spaces:

- Lab coats must be changed/swapped when moving between Characterization bay (White lab coat) and Live Cell Manipulation or Incubation bays (Blue lab coats.)

- Gloves must be removed before touching end door handles, phones, or personal devices.

- PPE must never be worn outside of designated laboratory areas.

# 5. Facility Entry and Exit Procedures

- Sanitize hands upon entry and before exiting

- No food, drink, gum, cosmetics, or personal items

- Clean and disinfect work surfaces before and after use with 70% ethanol

- Only approved materials may enter or leave the facility

- Use the shoe cleaner station before entering the facility (located near entrance) to reduce particulate contamination

- Use the adhesive sticky mat at the laboratory entrance to remove dust and debris from footwear

- Ensure proper use of PPE

# 6. Biosafety Practices

- Aerosol-generating procedures must be performed in a certified Class II BSC

- Minimize sharps; dispose in labeled shapes containers

- Biological materials cannot leave the facility without proper approval and packaging. Consult with the BioFoundry Engineer before transporting materials.

- Follow all BSL-specific safety rules

- Users are responsible for properly washing and decontaminating all materials and equipment used during their work

- Users are responsible for disposing of liquid biohazard waste in designated containers containing bleach. All liquid biohazard waste must be placed in this container to ensure proper chemical inactivation. The container will be emptied and managed by facility staff at the end of each day in accordance with facility biosafety procedures.

### Cell Culture Best Practices:

- Plan experiments and prepare materials in advance

- All supplies moved into the BSC and moved out of the BSC must be disinfected immediately prior to relocation

- Clean/disinfect BSC before and after use by wiping surfaces with 70% ethanol

- UV decontaminate BSC (~20 min), before and after use

- Work in organized, deliberate manner; avoid talking, coughing, or sudden movements inside BSC

- Only essential materials inside BSC

### Materials Returning to Incubators and Fridges:

- Clearly label containers with: user name, date, cell line/type

- Opened reagents, media, and other materials used for cell incubation must remain within BioFoundry to reduce risks of contamination.

- Ensure lids and caps are securely closed

### Incubator Guidelines:

- Minimize door opening time

- Label assigned lockers/compartments

- Avoid placing uncleaned/external materials in the incubators.

- Report contamination, spills, odors, or unexpected changes immediately

### Consumables Tracking:

- Use NEMO system for all consumables check out. (tips, plates, flasks, lockers)

  - Consumables that must be tracked are those sold by the BioFoundry, including flasks and culture plates, pipettes, tips, bottles, and filters. A detailed list of available items will be provided. These paid consumables will be stored in a dedicated cabinet designated exclusively for BioFoundry inventory.

Failure to track consumables may result in restricted facility access or retraining requirements.

### Shared Responsibility:

- Leave workspace equal or better than found

- Clean benches and common areas after use

- Properly dispose of waste

- Report dirty/abnormal equipment or abandoned materials

### Mandatory Core Equipment Training:

- Level 1: Basic operation, maintenance, routine experiments

- Level 2: Advanced or application-specific training

Unauthorized use may result in loss of access

### Proper Material Labeling:

- Username/ Lab

- Date

- Cell/reagent/sample identity

- Expiration

- Notes

Improperly labeled materials must not return to storage/incubators

# 7. Biological Materials Handling

- Only agents approved may be used

- Label all samples (identity, user/lab, date, biohazard designation)

- Secondary containment required for transport, plus absorbent material sufficient to completely absorb volume transported.

- Decontaminate materials before servicing/removal

Important: Only fixed or inactivated bacteria and yeast are allowed in Biocharacterization Area; live microbial cultures are prohibited unless under a separate approved protocol

# 8. Chemical Handling

- Follow all Standard Operating Procedures (SOPs) and wear appropriate PPE.

- Follow and abide by the University of Oregon Chemical Hygiene Plan: https://safety.uoregon.edu/sites/default/files/2023-10/chemical-hygiene-plan_11th_revision_2023.pdf

- Get help if you’re uncertain how to work with various chemicals

- Perform chemical work only at designated hoods/benches.

- Label all containers with:

- Full chemical name

- User name

- Date

- Use secondary containment for all liquid chemicals.

- Dispose of waste in clearly labeled chemical waste containers.

- Never mix chemicals unless the procedure specifically calls for it.

Buddy system required for work with dangerous chemicals or after-hours use.

# 9. Tool and Equipment Use

All equipment in the BioFoundry is shared and requires proper training before use.

General rules:

- Users may only operate equipment for which they have completed training.

- Equipment must be reserved and logged through the scheduling system.

- Equipment must be left clean and ready for the next user.

- Equipment problems or abnormal behavior must be reported immediately.

Training Levels:

Level 1 – Basic Training

- Equipment principles and operation

- Startup and shutdown procedures

- Routine user maintenance

- Basic experimental workflows

Level 2 – Advanced Training

- Specialized or non-routine applications

- Demonstrated independent operation and troubleshooting capability

# 10. Waste Management and Decontamination

- Solid biohazard waste must be disposed in red biohazard bags.

- Sharps must be placed in approved sharps containers.

- Liquid biological waste must be chemically disinfected.

- Autoclave use restricted to trained users.

# 11. Housekeeping and Laboratory Etiquette

- Keep workspaces clean and organized.

- Store materials in designated refrigerators and freezers.

- Clearly label all storage containers.

- Minimize noise and disruptions in shared spaces.

- Report unsafe or unsanitary conditions immediately.

Important:

Liquid Waste and Tip Disposal

All disposal of liquids and pipette tips containing liquid that are handled inside a Biosafety Cabinet (BSC) must first be discarded into a pre-decontamination container containing bleach at a final concentration equivalent to 10% of the total volume.

Liquid waste and pipette tips should be poured through a funnel equipped with a mesh strainer. The liquid fraction must pass through the strainer and be collected in the designated main liquid biohazard waste container containing bleach to ensure proper chemical inactivation. The retained pipette tips must then be disposed of in the appropriate solid biohazard waste container.

# 12. After‑Hours Use and Lone Worker Policy

The BioFoundry allows work outside normal hours, but users must evaluate safety before starting. Recently trained users were encouraged to begin experiments before 3:00 PM, so staff can assist with setup, operation, shutdown, and troubleshooting.

After-hours work may occur without staff present; in this case, the user and their PI assume full responsibility for experiment safety and equipment use. Users must follow all University of Oregon EHS policies regarding working alone. High-risk activities, such as handling hazardous chemicals, high-energy equipment, lasers, or biological hazards, should not be performed alone unless approved and risk assessed by the BioFoundry Engineer.

Manipulation of live cell cultures after hours is strongly discouraged; if necessary, the user and PI accept responsibility for associated risks. Users must ensure that after-hours work does not compromise personal safety, lab safety, equipment integrity, or University policy compliance.

## Permitted Activities (with PI Approval)

- Data analysis and computer work

- Operation of non‑hazardous instruments

- Handling of fixed, inactivated, or non‑infectious samples

- Manipulation of cell cultures and use of equipment for which the user has been trained

## Activities We Strongly Discourage

- Handling infectious materials

- Procedures generating aerosols

- Use of sharps with biological materials

- Biological spill response or biosafety cabinet maintenance

## Buddy System

- High‑risk or after‑hours work requires a buddy system or check‑in procedure.

- Users must inform colleagues before beginning work.

- Check‑ins via phone, text, or safety application may be required for lower risk activities.

- Buddy must be able to see you and assist in-person when high-risk activities are being performed after-hours and on weekends.

Working alone and/or after hours in BSL‑2 laboratories is discouraged due to safety risks.  See documentation attached.

# 13. Training, Records, and Documentation

- All training must be completed before facility access is granted.

- Training records must be maintained and updated annually.

- Documentation for SOPs, LSPs, and equipment training must be retained.

# 14. Enforcement

- Noncompliance may result in retraining or suspension of facility access.

- Serious violations may be reported to the PI, IBC, and EHS.

# 15. Violations and Enforcement

- Failure to follow any BioFoundry guideline may result in:

- Verbal or written warnings

- Temporary suspension of access

- Permanent removal from BioFoundry privileges

- Major safety violations will be reported to department leadership and university EH&S.